|  | Distance from light- 200 mm | Distance from light - 400 mm | Distance from light - 500 mm | Distance from light - 1000 mm | Distance from light - 1500 mm | # Notes / comments |
| --- | --- | --- | --- | --- | --- | --- |
| # Light Source | UVA / UVB microwatts/cm2 | UVA / UVB microwatts/cm2 | UVA / UVB microwatts/cm2 | UVA / UVB microwatts/cm2 | UVA / UVB microwatts/cm2 |  |
| # SUN (Natural outdoor light) Distance - Long way! |  |  |  | 13900 | 1700 | 11.00AM, September in Alice Springs, 32 degrees C – no clouds. |
| # High Intensity Discharge Lamps |  |  |  |  |  |  |
| # Metal Halide / GE “multi vapour” 250 watt |  |  | 900 / 0 |  | 100 / 0 | Open (no glass) white powder coated fixture. |
| # Mercury Vapour / EYE self ballasted - 250 watt |  |  | 1400 / 200 | 500 / 100 |  |  |
| # Mercury Vapour / EYE self ballasted - 160 watt | 1200 / 100 | 300 / 0 |  |  |  |  |
| # Quartz / Halogen Lamps |  |  |  |  |  |  |
| # Sylvania “Hi-spot” full voltage Halogen - 25º mini R50 40 watt |  | 200 / 200 |  |  |  | With front cover glass removed |
| # Sylvania “Hi-spot” full voltage Halogen - 10º mini R63 50 watt | 1000 / 1200 | 400 / 100 |  |  |  | With front cover glass removed |
| # Sylvania “Hi-spot” full voltage Halogen - 25º flood R63 50 watt | 500 / 300 | 0 / 100 |  |  |  | With front cover glass removed |
| # Sylvania “Hi-spot” full voltage Halogen - 10º spot R80 75 watt | 1900 / 1500 | 700 / 700 |  |  |  | With front cover glass removed |
| # Sylvania “Hi-spot” full voltage Halogen - 25º flood R80 75 watt | 600 / 400 | 100 / 100 |  |  |  | With front cover glass removed |
| # Sylvania “Hi-spot” full voltage Halogen - 10º spot R95 75 watt | 1300 / 1100 | 500 / 600 |  |  |  | With front cover glass removed |
| # Sylvania “Hi-spot” full voltage Halogen - 30º flood R95 75 watt | 400 / 500 | 100 / 100 |  |  |  | With front cover glass removed |
| # 12V Dichroics |  |  |  |  |  |  |
| # GE Dichroic MR16 precise constant colour 75 watt, 25º beam |  |  | 500 / 400 |  |  | Open face – no glass |
| # GE Dichroic MR16 precise constant colour 75 watt, 15º beam |  |  | 1600 / 800 |  |  | Open face – no glass |
| # Thorn Dichroic M58 50 watt, 38º beam |  |  | 300 / 100 |  |  | Glass front cover removed |
| # Fluorescent tubes |  |  |  |  |  |  |
| # Sylvania Reptistar 18 watt, 600 mm. NEW | 500 / 100 | 300 / 100 |  |  |  |  |
| # Sylvania Reptistar 18 watt, 600 mm. Used | 300 / 0 | 100 / 0 |  |  |  | 12 months use at 12 hrs/ day |
| # NEC Blacklight 20 watt, 600 mm T10 NEW | 3600 / 300 | 1300 / 100 |  |  |  |  |
| # NEC Blacklight 20 watt, 600 mm T10 Used | 1100 / 0 | 500 / 0 |  |  |  | 12 months use at 12 hrs/ day |

** Note that all light sources were tested at their full outputs.

In practical use, it may be necessary to “dim” down the light output of some of these light sources to achieve workable basking temperatures.